High-performance door Albany RR3000 Vision

Description:

High-performance door with transparent PVC sections, opening in vertical direction for interior and exterior use.

Side frames of edged galvanized steel profiles, springs for curtain tension and counterbalance in the side frames.

Door curtain: extensive, transparent PVC sections, framed by anodized aluminum profiles

Wind resistance: min. class 3-4, EN 12424.

Drive unit: Gear motor

Protection: IP 55

Control system:

Albany MCC, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all information for operation and service

control voltage 24 V DC.

Supply voltage:

3L/(N)PE 380/400/415/440V 50/60 Hz

Transformator necessary for 230/480/500V 50/60 Hz

Safety devices:

patented, contact-free pre-running safety photocell (transmitter/receiver)

door line photocell (transmitter/receiver)

drop down protection by integrated balancing system

Manual activation:

by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Opening / closing speed: up to max. 1.8/0.7 m/s

Size max.: (DW x DH) 5000 x 5000 mm

Make: Albany RR3000 GL

Options

Covers:

- Top roll cover made of anodized aluminum

- Motor cover made of anodized aluminum

Surface/material:

- Side frames RAL powdercoated

Miscellaneous:

- extended flexible seal

Actuators

Additional safety devices

Delivery

Installation incl. wiring