Aluminium High-performance door Albany RR3000 XXL

Description:

Aluminum High-performance door, opening in vertical direction, for interior and exterior use.

Self-supporting side frames of edged galvanized steel profiles, all steel components galvanized, springs for curtain tension and counterbalance in the side frame.

Door curtain of ecru anodized double walled aluminium lamellas, hingeless connected by means of flexible heavy belts, wear-less to wind up without to contact each other, each single lamella can be replaced easily, pre-fitted on the top roll.

Wind resistance: class 2-4 (EN 12424)

Drive unit: gear motor, pluggable electrical connections

Protection: IP 55

Control system:

Albany MCC, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service

Control voltage 24 V DC.

Supply voltage:

3L/(N)PE 380/400/415/440V; 50/60 Hz

Transformator necessary for 230/480/500V; 50/60 Hz.

Safety devices:

patented, contact-free pre-running safety photocell (transmitter/receiver)

stationary safety photocell (transmitter/receiver)

drop down protection by integrated balancing system

Manual activation: by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Opening / closing speed: up to max. 1.8/0.6 m/s

Size max.: (DW x DH): 8000 x 6000 mm; door surface max. 36 m2

Make: Albany RR3000 XXL

Options

Covers:

- Motor cover made of anodized aluminum

- Top roll cover

Surface/material:

- Side frames coloured

- Aluminum lamellas coloured

Miscellaneous:

- Aluminum lamellas with windows

- extended flexible seal

Actuators

Additional safety devices

Delivery

Installation incl. wiring