Crawford DL6010SM swingdock minidock

Product description:

Pneumatically-supported dock leveller with swing lip, specially developed for small working range

- Specially-designed loading system for uniform fleet or minor variations in height to ramp

- Dock leveller with flat bar frame easy to fit to existing ramps

- Parking position is supported by steel support constructions

- Steel support constructions with buffers RB/2 250x250x90mm

- The operation is supported by a gas spring

- Operated by one person who can lift the platform, swing out, and put the lip on the vehicle bed in one movement

- Gas spring with chain traction for easy and ergonomic opening!

- Free floating position

- Reinforced steel tear plates on dock and platform

- High quality gas spring

- Side plates such as toe guard to prevent crushing and shearing hazards

- Swing lip in steel, very flat design Bevelled lip 100 mm with 2° bend

- The dock leveller meets all requirements of the European directive EN 1398

Dimensions:

Nominal width	1250mm (40 kN) 2000mm (40 kN, 60 kN)

2200mm (40 kN, 60 kN)

Nominal length	700mm Depth in parking position 465mm

Surface:

- Surface painted in standard colour RAL 5010, thickness 80 µm

Options:

Frame

- Pit-frame (for installation in a pit)

Surface

- Surface painted in RAL 9005 / RAL 6005 / RAL 3002, thickness 80µm

- Surface painted with coat thickness 160µm

- Surface hot-dipped galvanised

Buffer designs

- Ramp installation

  - Steel support constructions with buffer RB/2 (250x250x90)

  - Steel support constructions with buffer RB (500x250x90)

  - Steel support constructions with buffer RB (500x250x90) and buffer guard plate

- Pit installation Frame P

  - Buffer design RB/2 (250x250x90) with welding plate

  - Buffer design RB (500x250x90) with welding plate

- Buffer design RB (500x250x90) with welding plate + front protection