Crawford DL6030C combidock

Product description:

Combidock extending bridge:

- Special construction for efficient loading and offloading of all sizes of vehicle, from truck to small transporter

- Integrated frame with wall anchors (T-frames)

- Electro-hydraulic operation with two lifting cylinders

- Lip extension in aluminium, in three sections, each with an extender cylinder

- Control unit with selector for "Big Truck" and "City van" operation

- "Big Truck" operation:

The lip is extended completely in one piece, total width 2000mm

- "City van" operation:

Only the central lip section is extended, width of central section 1000mm

When positioned on the vehicle the bearing strength is reduced to approx. 100kg

- Automatic, free floating position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class IP55

- The dock leveller meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

Dimensions:

Dimensions	Vertical Working range

| NL | LH | A | B - inside LH* | B - down to LP |
| --- | --- | --- | --- | --- |
| 3000 | 800 | 450 | 375 | 550 |
| 3500 | 900 | 560 | 360 | 650 |
| 4000 | 930 | 590 | 390 | 650 |

* When tail lift is in use Nominal width (NW) 2000mm.

Load capacity:

- "Big Truck" operation 60kN

- "City van" operation 20kN

Surface:

- Surface painted in standard colour RAL 5010, thickness 80µm

Hydraulics:

- Standard oil for temperature range -20°C - +60°C

Control unit:

- Voltage 400V 3-phase, 230V 3-phase , cable harness length 18m

- Impulse auto button to put the leveller back in parking position

Options:

Surface

- Surface painted in,RAL 3002, RAL 6005 or RAL 9005 thickness 80µm

- Surface painted with coat thickness 160µm

- Surface hot-dipped galvanised 80µm

Frame

- W-Frame (welded to pre-cast frame)

Hydraulics

- Low temperature oil for temperature range -30°C - +60°C

- Bio-oil (environmental protection)

Energy & Ergonomics

- EPDM gap sealing

- Slip protection and noise reduction coating

Multifunctional docking control for direct control of dock leveller, dock shelter and door with just one control unit

- Operating dock leveller and door