Crawford VL3110FCS vertical lifting fabric door

# Product description:

- Vertical lifting fabric door designed for industrial environments where doors are exposed to moisture, dust, very high or low temperatures or where the door opening is large.

- The unique design and structure offers durability, tightness, energy efficiency, operational reliability and minimum maintenance.

## Door leaf

- Door leaf made out of two layers of very strong vinyl-coated polyester fabric, separated by aluminium intermediate sections.

- Intermediate section which gives strength to the door leaf and which creates a buffer between the inside and outside fabric walls.

- Wind load is transferred to the vertical guide rails by the horizontal aluminium sections of the door leaf.

- The door leaf is available in 9 standard colours.

- Door leaf thickness: 100 mm.

- Aluminium guide rails.

## Bottom section

- The bottom section, made of steel and aluminium, is connected to the lifting belt via the safety arresters.

- The bottom section contains an optical safety edge and a rubber seal on the bottom edge that creates a seal between the door and the floor.

- Optical safety edge.

## Safety arresters and wind lock in stainless steel

- Safety arresters stop the door if the lifting belt becomes slack or, in an unlikely event, would break.

- Wind lock system locks the door when the door is closed.

## Surface treatment

- Steel components corrosion treatment: category 3 according to ISO 12944.2. Higher class on request. Aluminium, plastic, stainless steel, zinc electroplated steel (~10u). Fixing elements are mainly hot dip galvanized (FZV). Door leaf screws protected with Geomet.

## Door features

- Door range:

MIN: 1810 mm x (no MIN. height)

MAX (W x H): 6000 x 6000 mm (in combination). Other sizes available on request

- Opening speed: up to 1.5 m/sec.

- Temperature range: -35 °C to +70 °C

- Compliance with all operational and safety instructions in EU Directives and the standards of the European Standardisation Committee (CEN)

- Wind load (EN 12424)	Class 3-5 (0.7-1.6 kPa, depending on size).

- Water resistance (EN 12425)	Class 3, 0.11 kPa (for a closed door).

- Air permeability (EN 12426)	Class 2, 12 m3/(m2h).

- Thermal transmittance (EN

12428)

Depending on door size, specific data on request.

- Sound reduction (ISO 717)	15 dB Rw.

- Operating system: Electrical operator

# Dimensions:

x	mm (WxH)

# Options: Door leaf

  - Standard: Polyester, 1100 dtex with plasticised PVC coating

  - Option: Arctic fabric. For environmental temperatures down to -54°C

  - Option: Heat resistant fabric with silicone coating

  - Option: Heat resistant fabric with aluminium coating

  - Option: Heat resistant fabric with aluminium polyurethane coating

  - Option: Sound reduction fabric

  - Option: Security fabric

## Vision panels

800mm x 800mm

800mm x 1600mm

1300mm x 800mm

1300mm x 1600mm

## Fabric color

  - Tan (RAL 1001)

  - Red (RAL 3001)

  - Blue (RAL 5005)

  - Green (RAL 6009)

  - Grey (RAL 7004)

  - Anthracite (RAL 7016)

  - White (RAL 9016)

  - White aluminium (RAL 9006)

  - Translucent white

## Header box

  - Gear motor on inside of the building

  - Gear motor on outside of the building

  - Gear motor in the door opening

  - Insulated

  - Stainless steel cladding and motor casing

  - Self-supporting header box

## Operator

- Control system PLC-based

- Protection class, control cabinet IP65

- Protection class, safety boxes IP67

- Protection class, motor IP55

- Protection class, brake resistor IP51 (with protection)

- 3/phase 380-480V 50/60Hz power supply

- Control voltage 24V DC

- Fusing: 20 A / 32 A

- Heat and cold resistance, exterior: -35 °C to +70 °C

- Heat and cold resistance, inside control unit: -10 °C to +50 °C

- Power consumption: 2.2 or 4.1 kW

## Options

  - Radio remote control

  - “Open" (during closing process) if open button is used - door immediately stops and opens completely)

  - Pull switch for OPEN / CLOSE

  - External push-button, key switch

  - “One-button function” - opens or closes door, depending on actual status (e.g. radio, pull switch)

  - Additional safety (e.g. photo cell, radar) – door immediately opens completely

  - Free contacts

  - Photo cell

  - Reduced opening height

  - Radar

  - Magnetic loop

  - Warning lights

  - Emergency power switch

  - Interlocking door function

## Other supplementary equipment

  - Control cabinet in stainless steel

  - Heating element in the control cabinet

  - Emergency power switch

  - Clamp strip covers in the same colors as the fabric

  - Side jambs (if posts for installing the guide rails are not available)

  - Side jambs in stainless steel

- Wind deflectors (in combination with jambs)